Case Systems, Inc.

SECTION 12 32 16 – MANUFACTURED PLASTIC-LAMINATE-CLAD CASEWORK & MULTI-USE FURNISHINGS

GENERAL

ADMINISTRATIVE REQUIREMENTS

Pre-Installation Conference:

Attendance: [Architect,] [Owner,] [Contractor,] [Construction Manager,] [Design/Builder] installer, and related trades.

Review: Project conditions, manufacturer requirements, delivery, and storage, staging and sequencing, and protection of completed work.

SUBMITTALS

Action Submittals:

Shop Drawings: Illustrate products, installation, and relationship to adjacent construction.

Product Data: Manufacturer’s descriptive data and product attributes.

Samples:

[Selection samples.] [Verification samples.]

[Small scale base cabinet.]

Closeout Submittals:

Operation and Maintenance Data.

Quality Control Submittals:

Manufacturer qualification statement.

Comprehensive quality assurance standard documentation.

QUALITY ASSURANCE

Manufacturer Qualifications:

Manufacturer has a minimum of 10 years of experience of manufacturing architectural casework for similar projects, with a cataloged product line, and can show evidence of long-lasting financial stability.

Certified for chain of custody by third party group approved by Forest Stewardship Council (FSC).

Licensed under Architectural Woodwork Institute (AWI) Quality Certification Program (QCP).

Installer Qualifications: Minimum [2] [__] years’ experience in work of this Section.

Mockups:

Full-size [base cabinet] [wall cabinet] [[____] cabinet] [shelf unit], and [____].

Countertop, minimum [4] [__] feet long.

Approved mockups [may] [may not] remain as part of the work.

WARRANTIES

Fixed Casework: Manufacturer’s [1 year] [3 year] [5 year] [limited lifetime] warranty against defects in materials and workmanship,.

- Multi-Use furnishings including accessories: [1year] [3 year] warranty against defects in materials and workmanship.

PRODUCTS

MANUFACTURERS

Contract Documents are based on products by Case Systems, Inc., www.casesystems.com.

Substitutions: [Refer to Division 01.] [Not permitted.]

MATERIALS

Panel Core: Particleboard, ANSI A208.1, Grade M-2 or higher.

- **** OR ****

Panel Core: Moisture resistant particleboard, ANSI A208.1, interior grade moisture resistant, MR/FSC.

- **** OR ****

Panel Core: Recycled content, FSC Certified particleboard, ANSI A208.1, ULEF/FSC, M-2, containing recycled content, no added urea formaldehyde.

- **** OR ****

Panel Core: Recycled content, FSC Certified plywood, ULEF/FSC, no added urea formaldehyde.

- **** OR ****

Panel Core: FSC Certified plywood.

Surfacing: NEMA LD-3 and ANSI A161.2.

Vertical surface decorative grade: VGS.

General purpose decorative grade: HGS.

Cabinet decorative liner grade: CLS. White in color.

Non-decorative backer grade: BKH.

Decorative thermally fused laminate. White in color.

Tango decorative thermally fused laminate.

Chemical resistant decorative laminate.

Edge Banding:

PVC, applied utilizing hot melt adhesive and radiused by automatic trimmer.

Adhesive: PVA type, mechanically applied, ULEF, free from VOC’s.

FABRICATION

General Cabinet Body Construction:

Joinery: Concealed interlocking fasteners or dowel construction at manufacturer’s discretion.

Panels manufactured with balanced construction.

Structural components: 3/4 inch thick for bottoms and ends of cabinets, tops of tall cabinets and tops and bottoms of wall cabinets.

Fixed interior components: 3/4 inch thick for fixed shelves, dividers, and cubicle compartments.

Cabinet body exterior surfaces: [VGS.] [HGS.] [Decorative thermally fused.] [Tango decorative thermally fused.] [Chemical resistant laminate.] [As indicated on Drawings.]

Cabinet body interior surfaces:

- Closed cabinets: [VGS.] [HGS.] [CLS.] [BKH.] [Decorative thermally fused.] [Tango thermally fused.] [Chemical resistant decorative laminate.]

- Open cabinets: [VGS.] [HGS.] [CLS.] [BKH.] [Decorative thermally fused.] [Tango thermally fused.] [Chemical resistant decorative laminate.]

Laminate grain direction: [Combination grain.] [Vertical grain.] [Vertical grain match.]

Cabinet body front edge: [Nominally 1 mm thick PVC.] [Nominally 3 mm thick PVC.] [As indicated on Drawings.]

Cabinet body mounting: 3/4 inch thick stretchers, concealed by cabinet back.

Exposed laminate back panels: Finished 3/4 inch thick back panel when cabinet rear is exposed.

Cabinet backs: 1/2 inch thick, surfaced both sides, fully captured on both sides and bottom.

Stretchers:

¾ inch thick lower stretchers located behind back panel.

¾ inch thick intermediate stretchers located behind back panel.

Adjustable shelf hole pattern: 5 mm diameter holes spaced 1-1/4 inches on center for mounting adjustable shelves, hardware mounting, and replacement and relocation of cabinet components.

Shelf clips: adjustable, injection-molded clear polycarbonate, 5mm twin-pin for use with ¾” or 1” shelves on 32mm row boring, supporting a structural load up to 1200lbs per shelf (300lbs per clip).

Intermediate shelves: Fixed.

Base Cabinet Construction:

Sub-top: Solid 3/4 inch thick sub-top except at sink cabinets.

Split removable back panels for sink cabinets.

Wall Cabinet Construction:

Wall cabinets over 36 inches in width: Fixed intermediate partition.

Exterior bottoms: [Match standard interior.] [Match exterior surface.] [As indicated on Drawings.]

    - Tall and Wall Cabinet Top Edges: [Raw top edges.] [Nominally 1 mm thick PVC at top of end panels, stretchers and back.] [As indicated on Drawings.]

Tall, Wall, and Hutch Cabinet Exterior Top Finish: [Match standard interior.] [CLS to match interior.] [HPL to match exterior.] [As indicated on Drawings.]

Door Reveals:

Between doors and drawers: 3 mm.

Above drawer, base, wall, tall, and hutch doors: 3 mm.

    - Cabinet Toe Bases:

      - Material: [Particleboard.] [MR/FSC particleboard.] [ULEF/FSC particleboard.] [ULEF/FSC plywood.] [FSC plywood.]

      - Height: [96 mm.] [102 mm.] [152 mm.] [As indicated on Drawings.]

      - Type: [Attached.] [3/4 inch loose strips.] [No base.]

Drawer Fronts and Solid Doors: Surfaced on both sides.

Component materials: [HPL door and drawer front exterior and grade CLS on interior.] [HPL door interior and exterior; both sides match front surface color.] [As indicated on Drawings.]

Surfaces: [HPL Grade VGS.] [HPL Grade HGS.] [Thermally fused.] [Decorative thermally fused.] [Chemical resistant.] [As indicated on Drawings.]

Door and drawer front edges: [Nominally 1 mm thick PVC.] [Nominally 3 mm PVC.]

Drawer Boxes:

Material: [Particleboard.] [MR/FSC particleboard.] [ULEF/FSC particleboard.] [ULEF/FSC plywood.] [FSC plywood.] [Prefinished dovetail with hardwood veneer plywood.]

Core thickness: 1/2 inch thick.

Construction: Non-racking, non-deflecting platform bottom carried by bottom mount drawer slides on non-file drawers and side-mounted ball bearing slides on file drawers.

Drawer boxes with finished interiors: [Surface to match standard interior.] [Surface to match finished interior.] [Hardwood veneer plywood.]

Sides, backs, sub-fronts, and bottoms: 1/2 inch thick.

Top edges: Nominal 1 mm PVC, matching drawer color.

Box corners: Joined with hardwood dowels and glue.

Horizontal parting rails between drawers: 3/4 inch thick core, balanced surfaces.

File drawer box construction: Full height sides supporting heavy duty support rail.

Cabinet Doors:

Construction: Solid doors, 3/4 inch thick core.

Glazed doors:

Frame thickness: 3/4 inch.

Glazing panel thickness: 1/4 inch.

Glazing panel mounting: Set into door frame secured with removable stops.

- Operation: [Hinged.] [Sliding.]

Glazing type: [Clear acrylic.] [Safety glass.] [Smoked safety glass.] [Tempered glass.] [Smoked tempered glass.]

Frameless glazed doors:

Thickness: 1/4 inch.

Material: Tempered glass.

Operation: Sliding.

Glass edges: Radius ground and polished.

Sliding doors.

Upper track: Extruded aluminum, anodized finish.

Hardware: Two roller hanging brackets per door.

Bottom retainers.

Pocket doors: Self-closing, three-way adjustable door hinge with precision steel ball bearing slides, zinc plated.

Shelves:

Adjustable shelves: Finished on both sides.

Core material: [Particle board.] [MR/FSC particle board.] [ULEF/FSC particleboard.] [ULEF/FSC plywood.] [FSC Certified plywood.]

Thickness, in closed cabinets: [3/4 inch for shelves under [24] [27] [30] [33] [36] inches wide; 1 inch for shelved over [24] [27] [30] [33] [36] inches wide.] [1 inch.]

Thickness, in open cabinets: 1 inch thick, except for special use cabinets.

Edges in open cabinets: [Matching edges.] [Nominal 1 mm thick PVC] [Nominal 1 mm thick PVC] at front, [rear] [and] side] edges.

Edges in closed cabinets: [Matching edges.] [Nominal 1 mm thick PVC] [Nominal 3 mm thick PVC] at front, [rear] [and] side] edges.

Shelf surfaces in closed cabinets: [Match cabinet interior.] [Grade CLS on both sides.] [VGS HPL on both sides.] [HGS on both sides.] [Chemical resistant laminate.]

Shelf surfaces in open cabinets: [Match cabinet interior.] [Grade CLS on both sides.] [VGS HPL on both sides.] [HGS on both sides.] [Chemical resistant laminate.]

Setback for adjustable shelves: 15 mm from front.

Fixed shelves:

Thickness:

All shelves: [3/4 inch thick except 1 inch at open shelves.] [1 inch.]

Shelves under [24] [27] [30] [33] [36] inches wide: 3/4 inch.

Shelves over [24] [27] [30] [33] [36] inches wide: 1 inch.

Shelf surfaces in closed cabinets: Match interior.

Shelf surfaces in open cabinets: Match interior.

Wall shelves: [Thermally fused.] [Decorative thermally fused.] [Grade VGS.] [Grade HGS.] [Chemical resistant laminate.]

Wire shelves: White, plastic coated.

Hardboard shelves: 1/4 inch thick high density fiberboard with S2S surface finish.

Mobile Cabinets:

Mobile backs: 1 inch thick core.

Maximum load rating: 500 pounds.

Construction:

Core: 3/4 inch thick.

Core platform: 1 inch thick.

Edging for platform and mobile tops: 3 mm thick.

Mobile tops:

Overhang at front, sides and rear.

Surface materials: [VGS laminate.] [HGS laminate.] [Chemical resistant laminate.]

Surface material color: [____.] [To be selected.]

Sloped Tops: Separate unit from cabinet, set at 30-degree angle.

Lockers:

Match general casework construction except fixed shelved ¾ inch thick.

Locker tops: [Unfinished.] [Finished edge and surface.]

Plastic Laminate Countertops:

Surfacing: High-pressure decorative laminate, [grade HGS.] [Chemical resistant grade.] [As indicated on Drawings.]

Backing: Grade BKL.

Core thickness:

Solid: [3/4 inch.] [1 inch.] [1-1/8 inches.]

Built-up: 1-1/2 inches.

Joints: Secured with biscuits and tight joint fasteners.

Backsplashes: 4 inches high, thickness to match countertop thickness.

Edges: [3 mm thick for solid.] [Self-edge, same material as top.]

Epoxy Resin Tops: 1 inch thick with 1/32 inch chamfer on finished edges; provide drip grooves on underside at exposed edges.

Phenolic Resin Tops:

Thermosetting resins, homogeneously reinforced with wood-based fibers, manufactured under high heat and high pressure.

Non-porous, water resistant with integral anti-microbial properties.

Thickness: [3/4] [1] inch.

Edges: 1/8 inch thick, radius edge and drip groove on underside of top.

ACCESSORlES

    - Hardware:

Hinges: Five-knuckle type, reveal overlay, Institutional Grade per ANSI/BHMA A156.9; two per door except three per door for doors over 48 inches high.

- **** OR ****

Hinges: Concealed type, flush overlay, nickel plated, self-closing, minimum Grade 2 per ANSI/BHMA A156.9; two hinges for doors up to 48 inches high; three hinges for doors from 49 to 66 inches high; four hinges f\or doors 67 inches and higher.

Pulls: [Anodized aluminum wire type.] [Epoxy coated wire type.] [Brass core wire type.] [Stainless steel wire type.] [Semi-recessed type.] [Semi-recessed type, oversized.] [powder-coated steel, trapezoid shape to match SALTO furnishings]

Drawer slides:

Standard drawers: Full Extension, side mount, 100-pound rated, minimum Grade 1 per ANSI/BHMA A156.9.

- File drawers: Full extension, side mount, 100-pound rated, minimum Grade 1 per ASNI/BHMA A156.9.

Wall shelving hardware: Design: Heavy-duty wall shelving hardware. Finish: Anochrome.

Coat hooks: Zinc plated.

Closet rods: Chrome plated oval rod, 15 x 30 mm.

Mirrors: 8 x 10 inches.

Label holders: [None.] [At drawers.] [At doors.] [At doors and drawers.] [As indicated on Drawings.]

Locks:

Locations: [Drawers.] [Doors.] [As indicated on Drawings.]

Type: [Five-disc tumbler cam locks.] [Five pin lock, pin tumbler.] [Dead bolt style.] [Best - 5E.] [Olympus 100DR for doors and 200DW for drawers.]

Keying: [Key locks alike.] [Key locks differently by room and master key, minimum of 50 keying options.]

Lock cores: Removable, except at five pin locks.

Casters:  	[Model M, 6 inch diameter.] [Model U, 4 inch diameter nominal height.]

Bearings: Ball bearing, 360-degree swivel.

Wheels: Non-marring, [locking.] [non-locking.]

Catches:

Chain pulls: Zinc plated.

Chain stops: Zinc plated.

Elbow catches: Chrome plated.

Roller catches: Heavy-duty, spring-loaded.

Magnetic catches: White plastic housing.

Catch types:

          - [Magnetic at base and wall cabinets, single roller at tall cabinets.]

          - [Single roller at base, wall, and tall cabinets.]

          - [Magnetic at base and wall cabinets, two rollers at tall cabinets.]

          - [Single roller at base and wall cabinets, two rollers at tall cabinets.]

          - None for concealed hinges at base, wall, and tall cabinets.

Tote trays: [Impact resistant polystyrene with label holder, supported by polycarbonate rails] [ Gratnells totes to match SALTO furnishings]

Countertop Supports: Powder coated, formed metal.

SALTO® MULTI-USE FURNISHINGS FOR MAKER SPACES, STEM ROOMS, ESPORTS AND MORE.

Mobile Multi Use Cabinets

Cabinet Body:

Core Material: Panel Core: Particleboard, ANSI A208.1, Grade M-2 or higher.

Joinery: Concealed interlocking fasteners.

Panels manufactured with balanced construction.

Recessed Design with inset end panels to allow seamless integration with fixed casework and side-by side alignment.

Structural components: 1 inch thick for bottom, back panels, end panels and tops of cabinets.

Fixed interior components: 3/4 inch thick for fixed shelves, 1 inch thick for shelves over 36 inches wide, dividers, and cubicle compartments.

Cabinet body exterior surfaces: [VGS.] [HGS.] [Chemical resistant laminate.] [As indicated on Drawings.]

Cabinet body interior surfaces:

            - Closed cabinets: [VGS.] [HGS.] [CLS.] [BKH.] [Chemical resistant decorative laminate.]

            - Open cabinets: [VGS.] [HGS.] [CLS.] [BKH [Chemical resistant decorative laminate.]

Laminate grain direction: [Horizontal grain.] [Vertical grain.]

Cabinet body edge: Nominally 1 mm thick

Cabinet top and bottom edge: Nominally 3 mm thick PVC.

Epoxy powder coated 14 ga steel corner frame in recessed design.

Powder coat color: Stealth Charcoal Metallic

Peg board: epoxy powder-coated 14 ga steel in Dash Design hole pattern. Color: [  ] [to be selected from SALTO Standard color offering]

Casters: sinusoidal split wheel design

Meets or exceeds ANSI/BIFMA X5.9-2019 testing standards

Steel powder-coated whiteboard [not added] [added to the back]

Hardware:

Pulls: Trapezoid shape, powder-coated steel. Color: Stealth Charcoal Metallic

Hinges:   Concealed, flush overlay, nickel plated, self-closing, minimum Grade 2 per ANSI/BHMA A156.9; two hinges for doors up to 48 inches high; three hinges for doors from 49 to 66 inches high

Handles: powder-coated aluminum in matte nickel finish for mobile storage units taller than 36 inches.

Locks:

            - Locations: Doors.

            - Type: Five-disc tumbler cam locks. [

            - Keying: [Key locks alike.] [Key locks differently by room and master key, minimum of 50 keying options.]

            - Lock cores: Removable

Totes : Gratnells style according to manufacturer’s design. Color: translucent

Maximum load rating: 1,200 lbs. evenly distributed static load

Maker Space Table

Frame: Epoxy powder coated steel in recessed design including utility rail with Dash Design hole pattern. Powder-coat color:  Stealth Charcoal Metallic

Legs:  Epoxy powder-coated steel legs. Color:  [   ] [to be selected from SALTO Standard color offering]

Pegboard: powder coated steel in Dash Design hole pattern. Color: Stealth Charcoal Metallic

Top:

Mable Butcher Block, Size [30” x 60”] [30” x 72”]

- *** OR ***

High Pressure laminate on particle board core 1-1/8” thick with 3mm flexible PVC edge

            - Size [30” x 60]  [30 x 72]

            - Shape [rectangular] [one-sided cut-out]

            - Surface materials: [HGS Laminate] [Chemical Resistant Laminate]

            - Surface material color [    ] [ To be selected from SALTO standard offering]

            - Flexible PVC Edge [  ]  [ to be selected from SALTO standard offering]

Tote storage: [single bin tote shelf] [tote drawer slides]

Feet: [4 casters: sinusoidal split-wheel design] [4 leveler feet]

Accessories: [manufacturer’s included offering] [to be selected]

Meets or exceeds ANSI/BIFMA X5.5-2019 standards

Load Rating: 1,200 lbs. evenly distributed static load

Multi-Purpose Table;

Frame: Powder coated steel [with utility rail] [on sides] [on front and back] in Dash Design hole pattern. Color: Stealth Charcoal

Legs: Epoxy powder coated 18 ga steel. Color:  Silver Metallic Shimmer

Top:  High Pressure laminate on particle board core 1 1/8” thick with 3mm flexible PVC edge

Surface materials: [HGS Laminate] [Chemical Resistant Laminate]

Surface Material color [    ] [To be selected]

Size [24” x 48”] [24” x60”] [24” x 64”], [24” x 72”], [30” x 60”] [30”x 64”] [30” x 72’] 
[30” x 69” trapezoid only]

Shape: [Rectangular] [one-sided cut-out] [trapezoid]

Feet: [4 Casters: sinusoidal split-wheel design] [4 leveler feet]

Meets or exceeds ANSI/BIFMA X5.5-2019 standards

Load Rating; 500 lbs evenly distributed load

Maker Center Workstation:

Core Material: Panel Core: Particleboard, ANSI A208.1, Grade M-2 or higher.

Joinery: Concealed interlocking fasteners.

Panels manufactured with balanced construction.

Structural components: 1” inch thick for back panel bottoms and ¾” for end panels of cabinets.

Fixed interior components: 3/4 inch thick for fixed shelves, dividers, and cubicle compartments.

Cabinet body exterior surfaces: [VGS.] [HGS.] [Chemical resistant laminate.] [As indicated on Drawings.]

Cabinet body interior surfaces:

            - Closed cabinets: [VGS.] [HGS.] [CLS.] [BKH.] [Chemical resistant decorative laminate.]

            - Open cabinets: [VGS.] [HGS.] [CLS.] [BKH.] [Chemical resistant decorative laminate.]

Maximum load rating: 1200 lbs evenly distributed static load

Laminate grain direction: [Horizontal grain.] [Vertical grain.]

Cabinet body front edge: Nominally 3 mm thick PVC.

Epoxy powder-coated frame in recessed design.  Powder coat color: Stealth Charcoal Metallic Shimmer.

Top:

Mable Butcher Block, Size [30” x 60”] [48” x 54”]

- *** OR ***

High Pressure laminate on particle board core 1-1/8“ thick with 3mm flexible PVC edge

            - Size [30” x 60] [48 x 54]

            - Surface materials: [HGS Laminate] [Chemical Resistant Laminate]

            - Surface material color [   ] [ To be selected from SALTO standard offering]

            - Flexible PVC Edge [   ]  [ to be selected from SALTO standard offering]

Peg board: epoxy powder coated steel in Dash Design hole pattern.  Color: [   ] [to be selected from SALTO Standard color offering]

Hardware:

Pulls: Trapezoid shape, powder-coated steel. Color: Stealth Charcoal Metallic Shimmer

Hinges:   Concealed, flush overlay, nickel plated, self-closing, minimum Grade 2 per ANSI/BHMA A156.9; two hinges for doors up to 48 inches high; three hinges for doors from 49 to 66 inches high

Locks:

            - Locations: Doors.

            - Type: Five-disc tumbler cam locks.

            - Keying: [Key locks alike.] [Key locks differently by room and master key, minimum of 50 keying options.]

            - Lock cores: Removable

Casters: sinusoidal split-wheel design

Meets or exceeds ANSI/BIFMA X5.9-2019 testing standards

Wall Mount Panel System:

      - Wall Frame:  Epoxy powder coated steel in Dash Design hole pattern, Color: Stealth Charcoal (Color number)

Panel Options:

Pegboard:  Epoxy powder coated steel in Dash Design hole pattern, Color: Stealth Charcoal [32”x20”] [32”x 4”]

Whiteboard: Epoxy powder coated steel whiteboard, [32”x20”], [32”x 4”]

Shelves:  Epoxy powder coated steel,

Color:  [   ]  [To be selected from SALTO standard offering]

Shelf Load Rating:   100 lbs per shelf evenly distributed

EXECUTION

INSTALLATION

Install in accordance with manufacturer’s instructions and approved Shop Drawings.

- .

END OF SECTION